ESIOT Lab Internal 2

Raspberry Pi 3 — Sensor Interfacing Programs

Laboratory Record

Topics Covered

1. Interfacing LEDs — Single, Two, Three (Traffic Lights) with and without loops

2. Interfacing Buzzer — Separate, with LED (combined & alternate)

3. Interfacing Push Button with LED

4. Interfacing a Relay Module

5. Interfacing Rain Sensor

6. Interfacing Soil Moisture Sensor

7. Interfacing IR Sensor

8. Interfacing PIR Sensor

9. Interfacing Temperature & Humidity Sensor (DHT11)

# Experiment 1: Interfacing LEDs with Raspberry Pi

| AIM | To interface LEDs with Raspberry Pi GPIO and control them using Python (with and without loops). |
| --- | --- |

## 1A — Single LED (Without Loop)

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BOARD)

GPIO.setwarnings(False)

GPIO.setup(18, GPIO.OUT)

print("LED ON")

GPIO.output(18, GPIO.HIGH)

time.sleep(1)

print("LED OFF")

GPIO.output(18, GPIO.LOW)

time.sleep(1)

GPIO.cleanup()

| Output | Prints 'LED ON', turns LED on for 1 second, then 'LED OFF', turns LED off and cleans up. |
| --- | --- |

## 1B — Two LEDs (Without Loop)

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

led1 = 18

led2 = 23

GPIO.setup(led1, GPIO.OUT)

GPIO.setup(led2, GPIO.OUT)

print("LED1 on")

GPIO.output(led1, GPIO.HIGH)

time.sleep(1)

print("LED off")

GPIO.output(led1, GPIO.LOW)

print("LED2 on")

GPIO.output(led2, GPIO.HIGH)

time.sleep(1)

print("LED off")

GPIO.output(led2, GPIO.LOW)

GPIO.cleanup()

| Output | LED1 turns ON for 1s, then OFF. LED2 turns ON for 1s, then OFF. Sequential execution. |
| --- | --- |

## 1C — Three LEDs Traffic Light (Without Loop)

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

led1 = 18

led2 = 23

led3 = 24

GPIO.setup(led1, GPIO.OUT)

GPIO.setup(led2, GPIO.OUT)

GPIO.setup(led3, GPIO.OUT)

print("LED1 on")

GPIO.output(led1, GPIO.HIGH)

time.sleep(1)

print("LED off")

GPIO.output(led1, GPIO.LOW)

print("LED2 on")

GPIO.output(led2, GPIO.HIGH)

time.sleep(1)

print("LED off")

GPIO.output(led2, GPIO.LOW)

print("LED3 on")

GPIO.output(led3, GPIO.HIGH)

time.sleep(1)

print("LED off")

GPIO.output(led3, GPIO.LOW)

GPIO.cleanup()

| Output | LED1, LED2, LED3 each turn on and off in sequence for 1 second each. |
| --- | --- |

## 1D — Traffic Light with Loop (While True)

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

leds = [18, 23, 24]   # Red, Yellow, Green

for pin in leds:

GPIO.setup(pin, GPIO.OUT)

try:

while True:

# Red ON

GPIO.output(18, GPIO.HIGH)

GPIO.output(23, GPIO.LOW)

GPIO.output(24, GPIO.LOW)

print("Red ON (Stop)")

time.sleep(2)

# Yellow ON

GPIO.output(18, GPIO.LOW)

GPIO.output(23, GPIO.HIGH)

GPIO.output(24, GPIO.LOW)

print("Yellow ON (Ready)")

time.sleep(1)

# Green ON

GPIO.output(18, GPIO.LOW)

GPIO.output(23, GPIO.LOW)

GPIO.output(24, GPIO.HIGH)

print("Green ON (Go)")

time.sleep(2)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | Simulates a traffic light: Red (2s) → Yellow (1s) → Green (2s), repeating until Ctrl+C. |
| --- | --- |

# Experiment 2: Interfacing Buzzer, LED and Push Button

| AIM | To interface a buzzer, LED, and push button with Raspberry Pi and control them separately and in combination. |
| --- | --- |

## 2A — Buzzer Only

import RPi.GPIO as GPIO

from time import sleep

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BOARD)

buzzer = 3

GPIO.setup(buzzer, GPIO.OUT)

try:

while True:

GPIO.output(buzzer, GPIO.HIGH)

print("BEEP")

sleep(1)

GPIO.output(buzzer, GPIO.LOW)

print("OFF")

sleep(1)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | Buzzer beeps ON for 1 second, then OFF for 1 second, repeating continuously. |
| --- | --- |

## 2B — LED + Buzzer (Combined: ON/OFF together)

import RPi.GPIO as GPIO

from time import sleep

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BOARD)

buzzer = 3

led = 5

GPIO.setup(buzzer, GPIO.OUT)

GPIO.setup(led, GPIO.OUT)

try:

while True:

GPIO.output(buzzer, GPIO.HIGH)

GPIO.output(led, GPIO.HIGH)

print("BEEP + LED ON")

sleep(1)

GPIO.output(buzzer, GPIO.LOW)

GPIO.output(led, GPIO.LOW)

print("OFF")

sleep(1)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | Both LED and buzzer turn ON simultaneously for 1s, then OFF for 1s. |
| --- | --- |

## 2C — LED + Buzzer (Alternate: one ON while other OFF)

import RPi.GPIO as GPIO

from time import sleep

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BOARD)

buzzer = 3

led = 5

GPIO.setup(buzzer, GPIO.OUT)

GPIO.setup(led, GPIO.OUT)

try:

while True:

GPIO.output(buzzer, GPIO.HIGH)

GPIO.output(led, GPIO.LOW)

print("Buzzer ON | LED OFF")

sleep(1)

GPIO.output(buzzer, GPIO.LOW)

GPIO.output(led, GPIO.HIGH)

print("Buzzer OFF | LED ON")

sleep(1)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | Buzzer ON & LED OFF for 1s, then Buzzer OFF & LED ON for 1s, alternating. |
| --- | --- |

## 2D — Push Button controlling LED

import RPi.GPIO as GPIO

from time import sleep

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BOARD)

led = 5

button = 7

GPIO.setup(led, GPIO.OUT)

GPIO.setup(button, GPIO.IN, pull_up_down=GPIO.PUD_DOWN)

try:

while True:

if GPIO.input(button) == GPIO.HIGH:

GPIO.output(led, GPIO.HIGH)

print("Button Pressed -> LED ON")

else:

GPIO.output(led, GPIO.LOW)

print("Button Released -> LED OFF")

sleep(0.1)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | When button is pressed, LED turns ON and prints message. Releasing button turns LED OFF. |
| --- | --- |

# Experiment 3: Interfacing a Relay Module

| AIM | To interface a relay module with Raspberry Pi to control high-power devices using GPIO output pins. |
| --- | --- |

## 3A — Single Relay (ON/OFF)

import RPi.GPIO as GPIO

from time import sleep

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BOARD)

relay = 18

GPIO.setup(relay, GPIO.OUT)

try:

while True:

GPIO.output(relay, GPIO.LOW)   # Relay ON (active-low module)

print("Relay ON")

sleep(2)

GPIO.output(relay, GPIO.HIGH)  # Relay OFF

print("Relay OFF")

sleep(2)

except KeyboardInterrupt:

GPIO.cleanup()

| Note | Most relay modules are ACTIVE LOW: GPIO.LOW = Relay ON, GPIO.HIGH = Relay OFF. |
| --- | --- |

## 3B — Four-Channel Relay (Alternating Pattern)

import RPi.GPIO as GPIO

from time import sleep

GPIO.setmode(GPIO.BOARD)

GPIO.setup(18, GPIO.OUT)

GPIO.setup(22, GPIO.OUT)

GPIO.setup(24, GPIO.OUT)

GPIO.setup(26, GPIO.OUT)

try:

while True:

GPIO.output(18, GPIO.HIGH)

GPIO.output(22, GPIO.LOW)

GPIO.output(24, GPIO.HIGH)

GPIO.output(26, GPIO.LOW)

sleep(2)

GPIO.output(18, GPIO.LOW)

GPIO.output(22, GPIO.HIGH)

GPIO.output(24, GPIO.LOW)

GPIO.output(26, GPIO.HIGH)

sleep(1)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | Channels 18 & 24 activate for 2s, then 22 & 26 activate for 1s, repeating in a loop. |
| --- | --- |

# Experiment 4: Interfacing Rain Sensor

| AIM | To detect the presence of rain using a rain drop sensor connected to the Raspberry Pi GPIO. |
| --- | --- |

## Circuit

Connect the digital output (DO) of the rain sensor to GPIO pin 18 (BCM). The sensor outputs LOW when rain is detected and HIGH when dry.

## Code

from time import sleep

from gpiozero import InputDevice

no_rain = InputDevice(18)

while True:

if not no_rain.is_active:

print("Rain")

else:

print("No Rain")

sleep(1)

| Output | Prints 'Rain' when rain is detected (sensor active / LOW), and 'No Rain' when dry — checked every 1 second. |
| --- | --- |

# Experiment 5: Interfacing Soil Moisture Sensor

| AIM | To detect soil moisture levels using a capacitive/resistive soil moisture sensor with Raspberry Pi. |
| --- | --- |

## Circuit

Connect the digital output (DO) of the soil moisture sensor to GPIO pin 4 (BCM). LOW = wet soil (moisture detected), HIGH = dry soil.

## Code

import RPi.GPIO as GPIO

import time

c = 4

GPIO.setmode(GPIO.BCM)

GPIO.setup(c, GPIO.IN)

def callback(c):

if GPIO.input(c):

print("moisture not detected")

else:

print("moisture detected")

GPIO.add_event_detect(c, GPIO.BOTH, bouncetime=300)

GPIO.add_event_callback(c, callback)

try:

while True:

time.sleep(0.5)

except:

GPIO.cleanup()

| Output | Uses event-based detection. Prints 'moisture detected' when soil is wet, 'moisture not detected' when dry. Responds to state changes instantly via callback. |
| --- | --- |

# Experiment 6: Interfacing IR Sensor

| AIM | To detect obstacles using an infrared (IR) sensor module connected to Raspberry Pi GPIO. |
| --- | --- |

## Circuit

Connect the digital output of the IR sensor to GPIO pin 18 (BCM). Output is LOW (0) when no object is detected, HIGH (1) when an object interrupts the IR beam.

## Code

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setup(18, GPIO.IN)

try:

while True:

i = GPIO.input(18)

if i == 0:

print("No interrupt")

time.sleep(0.1)

elif i == 1:

print("Interrupted")

time.sleep(0.1)

except KeyboardInterrupt:

GPIO.cleanup()

| Output | Prints 'No interrupt' when path is clear and 'Interrupted' when an object is detected. Polls every 100ms. |
| --- | --- |

# Experiment 7: Interfacing PIR Sensor

| AIM | To detect human/animal motion using a PIR (Passive Infrared) motion sensor and trigger an LED alarm output. |
| --- | --- |

## Circuit

PIR sensor OUT → GPIO 11 (BCM) as input. LED/buzzer → GPIO 3 (BCM) as output.

## Code

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(11, GPIO.IN)

GPIO.setup(3, GPIO.OUT)

try:

while True:

i = GPIO.input(11)

if i == 0:

print("No intruder")

GPIO.output(3, 0)

time.sleep(0.5)

elif i == 1:

print("Intruder detected")

GPIO.output(3, 1)

time.sleep(0.5)

except:

GPIO.cleanup()

| Output | Prints 'No intruder' and sets output LOW when no motion detected. Prints 'Intruder detected' and sets output HIGH when PIR is triggered. |
| --- | --- |

# Experiment 8: Interfacing Temperature & Humidity Sensor (DHT11)

| AIM | To read temperature and humidity data from a DHT11 sensor using the Adafruit_DHT library on Raspberry Pi. |
| --- | --- |

## Prerequisites

Install the library before running:

pip install Adafruit_DHT

## Circuit

Connect DHT11 DATA pin to GPIO 4 (BCM) via a 10kΩ pull-up resistor to 3.3V. The sensor type constant for DHT11 is 11.

## Code

import sys

import Adafruit_DHT

import time

while True:

humidity, temperature = Adafruit_DHT.read_retry(11, 4)

print("Temperature", temperature)

print("Humidity", humidity)

time.sleep(1)

| Output | Continuously prints temperature (°C) and humidity (%) values every 1 second. Uses retry logic to handle read failures from the DHT11 sensor. |
| --- | --- |

| Note | read_retry(11, 4) — '11' is DHT11 sensor type, '4' is the GPIO pin number (BCM). For DHT22, use 22 instead of 11. |
| --- | --- |

# Common Errors & Fixes

| File is unwritable | Open nano with sudo: sudo nano filename.py |
| --- | --- |

| KeyboardInterrupt not working | Ensure 'except KeyboardInterrupt' is spelled correctly and GPIO.cleanup() is called |
| --- | --- |

| RuntimeError: GPIO already in use | Add GPIO.setwarnings(False) before GPIO.setup() |
| --- | --- |

| GPIO.BCM vs GPIO.BOARD | BCM uses chip numbers (e.g., GPIO18). BOARD uses physical pin numbers (e.g., Pin 18). Be consistent! |
| --- | --- |

| DHT11 read fails | Use Adafruit_DHT.read_retry() — it tries up to 15 times automatically |
| --- | --- |

| Relay not switching | Most relay modules are active-low: LOW = ON, HIGH = OFF |
| --- | --- |

| PIR false triggers | Add time.sleep() after detection to avoid bouncing. Adjust sensitivity pot on sensor |
| --- | --- |